# Water Level Sensor

Index

Project Introduction

Executive Summary

Bried (Motivation and Problem Statement)

Methodology and design approach

Implementation and system design

Result

Conclusion

Recommendation and Future for Aliaxis

List of Use Component

Approximate cost per water sensor(?)

Challenges and Solution

Reference

Executive Summary

Aliaxis commissioned a prototype water-level sensing system to monitor farm tank levels remotely. The prototype must deliver accurate and stable level measurements, operate without RF emissions, support a specified transmission range between tank and shed, and offer long battery life for remote deployments. The final deliverable includes a probe design, a transmitter, and a receiver, optimized for reliability, low cost, and minimal maintenance.

Brief — Motivation and Problem Statement

Reliable access to water is critical for farm operations, particularly in dry seasons. Aliaxis requested a non-RF water level sensor to avoid regulatory testing and potential interference, while ensuring consistent, accurate readings that are not affected by environmental noise or other factors. The system must provide sufficient communication range to reach a distant shed, and it must be energy-efficient to minimize battery replacements in remote locations. This project aims to design a probe, transmitter, and receiver that meet these constraints, delivering a stable, low-cost, low-maintenance solution for farm water monitoring.

Methodology and design approach

Probe Design  Tx Design  Rx Design

Probe Design:

Water tank height long Polyvinyl Chloride (PVC) tube with 20mm diameter. The probe design to measure the water level by using the concept of change of electric field across different dielectric materials (different permittivity). Figure 1 below shows the detailed design of the probe. In this phase the difference in water level is represented by the change of capacitance.

PoC

PVC as the probe tube material because it has better accuracy and stability compared to Silicone Rubber.

Table:

Calculation:

Transmitter Design:

- No RF emission

- Greater Tx Range

- Battery Life

Implementation and system design

Result

- Accessible Tx range

Conclusion

Recommendation and Future for Aliaxis

- What changes can make to have even better design (increase Tx range?)

List of Use Component

Approximate cost per water sensor(?)

Challenges and Solution

Reference